The Importance of Understanding Vortex Induced Vibration

Vortex-induced-vibration (VIV) plays an important role in the design of many different types of engineered systems.  From bridges, to aircraft, to tall buildings, to antennas and utility lines, VIV is a means by which structures can fail.

Most structural failures result from either inadequate design, inadequate materials of construction, or loading that was not anticipated by the designer.  In many cases, two or more of these factors contribute to failure.

Many structures are designed to have very little or no response (e.g., deformation) under conditions thought to be present in anticipated use.   A key problem, however, is that VIV results from forces due to flow of a fluid (typically air or water), which for many years were not accounted for in many structural designs.  As discussed in <<VideoXX>, alternate shedding of vortices from opposite sides of the cylinder gives rise to a time-fluctuating force on the cylinder.  This structural loading therefore depends on time, and requires a different type of analysis than is needed to deal with other kinds of loading, such as earthquake loading.

Some examples of structures that have either failed, or which undergo significant deformation due to VIV are highlighted in the videos to which links are provided below.

Tacoma Narrows Bridge (https://www.youtube.com/watch?v=KVc7oBKzq9U)

The collapse of the Tacoma Narrows Bridge in 1940 was a dramatic example of how VIV could contribute to destruction of a large civil structure.  Theodor von Kármán, who in 1912 first explained the vortex shedding shown in <VideoXX>, played an important role in the post-failure investigation.

Building sway (https://www.youtube.com/watch?v=rlpUhgfEZPU)

Tall "narrow" buildings are known to deform in high winds, and it is important that the resulting motion not lead to structural collapse.  No tall building has collapsed due to wind loading, but a number of other tall civil structures have.  Examples include the 1989 collapse of 195 m (640 ft) tall "solar chimney" in Spain, and the collapse of numerous television and radio transmission towers up to 609 meters (1998 ft) high (see https://en.wikipedia.org/wiki/List_of_catastrophic_collapses_of_broadcast_masts_and_towers)

Aeroelasticity in aerodynamic systems (https://www.youtube.com/watch?v=QD3G86dfj6Y)

As aircraft move through the atmosphere, they are subjected to a multitude of dynamic forces, namely lift and drag. Sometimes these forces, under the right conditions, create VIV in the wings of the aircraft, more commonly referred to in aeronautics as “aeroelastic flutter”. This phenomenon is very serious, and can cause airplanes to shear themselves apart midflight if not accounted for in the design, as shown in the video above. The conditions that allow for or modify VIV in aircraft are very complex and difficult to predict analytically beforehand. It is because of this difficulty that VIV must be looked for and found through rigorous and often painstaking testing of new aircraft and airfoils being designed.

“Cable Galloping” (https://www.youtube.com/watch?v=8PaHcoiFUOg)

Similar to aeroelastic flutter, cable galloping (also known as conductor gallop) occurs due to the interaction of lift and drag forces being exerted on power transmission lines during windy days with the elasticity and other physical properties of the cable. If left uncontrolled, excessive cable galloping can cause severe damage to power lines, even causing them to snap and fall to the ground.

The relationship between the Karman vortex street and the VIV that led to failure of the bridge has been the subject of some debate.  See Steven Ross, et al. "Tacoma Narrows 1940." In Construction Disasters: Design Failures, Causes, and Prevention. McGraw Hill, 1984, pp. 216–239, and <<some YT video>>